Reduced order Model Predictive Control scheme applied to Binary Distillation Column process

Abstract:

Introduction:

Mathematical Modeling

The plant nominal capacity is 130,000 tons of raw condensate per year based on 24 operating hours per day and 350 working days per year. The plant equipment is specified with a design margin of 10% above the nominal capacity and turndown ratio of 50%. Hence, the raw condensate feed rate for the plant is determined as follows:

- The feed mass rate of the plant: Fmass  15.47619 (tons/hour)

- The gas percentage in the feed flow:  38%

- The internal vapor flow  selected by empirical:   28%

- The feed stream (m3/h) with the density =0.670 (ton/m3) is calculated as,

Composition  in the liquid and  in the vapor phase of the feed are obtained by solving the flash equations:

Solving flash equation with the relative volatility (  5.68 ),   0.26095; =0.66728

Table: Nominal values for process parameters of distillation column process

| Variable | Stream | Molar flow | Unit |
| --- | --- | --- | --- |
| MB | Liquid holdup in the column base | 31.11 | Kmole |
| M | Liquid holdup on a tray | 5.8 | Kmole |
| MD | Liquid holdup in the reflux drum | 13.07 | Kmole |

Table: Stream data for distillation column process

| Stream | Formular | % | Volume (m3/h) | Density (ton/m3 ) | Mass (ton/h) | Molar (kg/kmole) | Molar flow (kmole/h) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vapor rate in feed VF | cF+4 | 42 | 9.7015 | 0.591 | 5.7336 | 58.2 | 98.5152 |
| Liquid rate in feed LF | 100 - cF | 58 | 13.3973 | 0.726 | 9.7264 | 93.3 | 104.2491 |
| Internal vapor rate V | Vf | 28 | 6.4677 | 0.598 | 3.8677 | 58.3 | 66.3407 |
| Internal liquid rate L | Vf+4 | 32 | 7.3916 | 0.615 | 4.5458 | 60.1 | 75.6380 |
| Distillate flow rate D | cF | 38 | 8.7775 | 0.576 | 5.0554 | 54.5 | 92.7597 |
| Bottom flow rate B | 100 - cF | 62 | 14.3213 | 0.727 | 10.405 | 93.8 | 110.9235 |

V, Y1

V, Ym-1

V, Ym

V, Yf-1

V, Yf

VF , YF

(V+VF) , YN

(V+VF) , Yn

(V+VF) , Yn-1

(V+VF) , Yf+1

(L+Lf) , X2

(L+Lf) , Xm

(L+Lf) , Xm+1

(L+Lf) , Xf

LF , XF

L , Xf+1

L , Xn

L , Xf+2

M

f

M

f+1

m

M

1

MB

B, X1=XB

M

n

L , Xn+1

L , XN+1

MD

N+1

D, XN+1=XD

Condenser (N+1=16):

Tray n (n=10 to N=15):

Tray above the feed flow (f+1=9):

Tray below the feed flow (f=8):

Tray m (m=2 to 7):

Re-boiler (Tray 1):

Linearization of Model

Elements of the linearization matrices by Jacobian method,

;                  ;

;

Substituting values:

= L = 75.6380				LF=104.2491

= V= 66.3407				VF=98.5152

D=92.7597					B=110.9235

MD=13.07					MB=31.11

M=5.8						= 5.68

XF=0.26095					YF=0.66728

## Table : Steady state values of  xn, yn, Kn on each tray using ode45 solver

| Trays | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xn | 0.9953 | 0.9739 | 0.9228 | 0.8168 | 0.6500 | 0.4746 | 0.3534 | 0.2920 |
| yn | 0.9992 | 0.9953 | 0.9855 | 0.9620 | 0.9134 | 0.8369 | 0.7564 | 0.7008 |
| Kn | 0.1774 | 0.1839 | 0.2008 | 0.2442 | 0.3476 | 0.5475 | 0.8063 | 1.0143 |
| Trays | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| xn | 0.2727 | 0.2696 | 0.2622 | 0.2451 | 0.2104 | 0.1539 | 0.0883 | 0.0365 |
| yn | 0.6805 | 0.6771 | 0.6687 | 0.6484 | 0.6022 | 0.5082 | 0.3549 | 0.1770 |
| Kn | 1.0961 | 1.1102 | 1.1454 | 1.2320 | 1.4418 | 1.9192 | 2.8441 | 4.1441 |

PID Controller design

Table : PID Controller design

| Compositions | Full Order system |  |  | Reduced Order system |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | P | I | D | P | I | D |
| Bottom product | 108.5052 | 207.1671 | -- | 182.3268 | 220.3698 | -- |
| Top product | 223.0076 | 159.0710 | -74.6747 | 212.9030 | 126.1064 | -135.5175 |

Figure: Control response of PID controller of a full order binary distillation column

Figure: Control response of PID controller of a reduced order binary distillation column

TABLE: PID controller performance comparison of binary distillation column

| Compositions | Full order system |  |  | Reduced order system |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ISE | IAE | MSE | ISE | IAE | MSE |
| XB | 0.4141 100 | 20.8925100 | 0.0836100 | 0.3780 100 | 18.9139100 | 0.0757100 |
| XD | 0.0613 100 | 8.9406100 | 0.0358100 | 0.0757 100 | 9.3724100 | 0.0375100 |

Predictive Control design:

## GPC with transfer function model

where, A1, A2, B11, B12, B21, B22, C1and C2 are polynomials defined as,

The operator  is defined as . The variables y1(k), y2(k),  u1(k), u2(k),  e1(k) and e2(k)  are output, input and noise. The noise is supposed to be white noise random signal with zero mean.

Polynomials A1, A2 and B11, B12, B21, B22 are denominator and numerator of transfer function respectively. Polynomial C1and C2is disturbance model polynomial or can be seen as filtering polynomial. Rearranging the terms from ,

The difference equation for step k+1 for the first output is obtained as,

The difference equation for step k+2 is obtained as,

Similarly, the difference equation for step k+N for the first output is obtained as,

Assuming,

By separating variables into future and past (together with present) and rewriting (7), (8), (9) in matrix form we get,

Similarly, difference equation for step k+N for the second output is obtained as,

Assuming,

By separating variables into future and past (together with present) and rewriting in matrix form we get,

By separating first and second output variables into a single matrix we get,

where,

A, B, Ap, Cp, Bp matrices are filled with corresponding coefficients from eq.

By multiplying A-1 on both sides in (10), we get

The prediction of the output is separated into two parts – free response and forced response. The simplified representation of the combined predicted output is as shown,

## Cost funtion

The cost function in (3) can be represented in matrix format as,

where, R and Q are diagonal matrix with diagonal elements r and q respectively.

Substituting, from (11) or (14),

where,

The control law for unconstrained MPC cost function minimization is,

First two rows and  in L corresponds to control action at time k,

## Constraints

In a long range predictive control the controller has to anticipate constraint violation and correct them in an appropriate way. The output constraints are mainly due to safety reasons and must be controlled in advance, because output variables are affected by process dynamics. Input / manipulated variables can always be kept in bound, by limiting the control action to an appropriate value satisfying the constraints. The two input and two output constraints are,

The implementation of MPC with constraints involves the minimization of a quadratic cost function subject to linear inequalities: Quadratic Programming (QP) – in MATLAB quadprog function from optimization toolbox. The Hessian matrix M and gradient vector m are obtained from (16).  The inequality and equality constraints is given by,

Step by step implementation procedure for transfer function model is described below:

1) Define system continuous time process model and initialize sampling time, constraints, polynomial filter coefficients, weighting coefficient for control error and control increment.

2) Discretize the system transfer function model.

3) Compute prediction matrices (G, F).

4) Compute H, g and other matrices for cost function by quadratic programming.

5) MPC iteration, at each time instant k recursively,

- Compute  and  (simulated output or measured output).

- Find prediction error  and .

- Find  and from cost function by considering constraints using quadratic programming.

- Determine predicted output,and .

Figure: Control response of controller with linear model of a full order binary distillation column

Figure: Control response of controller with linear model of a reduced order binary distillation column

Figure: Control response of controller with non-linear model of a full order binary distillation column

Figure: Control response of controller with linear model of a reduced order binary distillation column

Figure: Bode plot for full order and reduced order model of a binary distillation column

Multiple positive and negative set-point steps were applied for bottom product XB (output 1) at time t=25 hr, 175 hr and t=100 hr respectively. Similarly, multiple positive and negative set-point steps were also applied for top product XD (output 2) at time t=135 hr, 220 hr and t=50 hr respectively.  Response of the controller with the transfer function for linear and nonlinear model is in Fig. . The setpoints for bottom product XB and top product XD are plotted with the solid line and the dotted line shows the linear and nonlinear model responses.

Figure: Pole- zero mapping for full order binary distillation column

TABLE 1. Model Predictive Controller parameters for full order and reduced order binary distillation column

| Controller parameter | Value |
| --- | --- |
| Sampling time | 1 s |
| Start of costing horizon | 1 |
| End of costing horizon | 40 |
| End of control horizon | 40 |
| Weighting coefficient for control error |  |
| Weighting coefficient for control increments | 1 |

Full order system poles:  -99.8905, -80.0087, -69.7103, -62.3181, -50.1214, -42.2187, -34.012, -29.9989, -24.4728, -17.9833, -15.0707, -10.7188, -7.8889, -5.5823, -1.5563, -0.4098.

Reduced order system poles:  -0.3996, -1.3040.

TABLE 3. MPC controller performance comparison of full order with reduced order linear and nonlinear model of binary distillation column

| Models |  | Full order system |  |  | Reduced order system |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ISE | IAE | MSE | ISE | IAE | MSE |
| Linear | XB | 3.3297 10-1 | 9.1201100 | 0.0013100 | 3.329610-1 | 9.1209100 | 0.0013100 |
|  | XD | 2.4766 102 | 2.4882102 | 0.9906100 | 2.4765102 | 2.4883102 | 0.9906100 |
| Non-linear | XB | 3.331510-1 | 9.1200100 | 0.0013100 | 3.332310-1 | 9.1204100 | 0.0013100 |
|  | XD | 2.4765102 | 2.4883102 | 0.9906100 | 2.4766102 | 2.4883102 | 0.9906100 |

TABLE 4. Comparison of time elapsed for full order with reduced order linear and nonlinear model of binary distillation column

| Models | Time elapsed (s) |  |
| --- | --- | --- |
|  | Full order system | Reduced order system |
| Linear | 5.2713 | 4.3387 |
| Non-linear | 5.8200 | 4.5376 |

TABLE 5. Comparison of matrix dimensions and memory used for full order with reduced order constrained MPC of binary distillation column

| Vector | Full order system |  | Reduced order system |  |
| --- | --- | --- | --- | --- |
|  | Matrix dimension | Memory (Bytes) | Matrix dimension | Memory (Bytes) |
| F | 80132 | 84480 | 4020 | 12800 |
| X | 1321 | 1056 | 201 | 160 |