OPTIMIZING OF PHOSPHORUS DOSAGE IN AUTOTROPHIC DENITRIFICATION OF THIOBACILLUS DENITRIFICANS IN BATCH REACTORS

Blažková Z., Slehová E., Trousil V., Muselíková J., Palarčík J., Slezák M., Cakl J.

Institute of Environmental and Chemical Engineering, Faculty of Chemical Technology, University of Pardubice, Studentská 573, 532 10, Pardubice
Zuzana.Blazkova@upce.cz

## Abstract

Thiobacillus (T.) denitrificans is widely distributed microorganism, which is not toxic and pathogenic. It belongs to group of chemolithoautotrophic and facultative anaerobic bacteria.  It can obtain energy by the oxidation of elemental sulfur and certain other sulfur-containing inorganic compounds while electrons are released. Under anoxic conditions, nitrate and nitrite ions are reduced by these electrons. It is environmentally important – T. denitrificans can be used in processes of alternative methods of removing nitrates from contaminated water.

In this work, we focused on the optimal dosage of phosphorus to the reactor for autotrophic denitrification using T. denitrificans. The optimal dosage of phosphorus is one of the key factors affecting growth of bacteria cells and does not lead to secondary contamination of treated water. All experiments were done in batch reactors and the elemental sulfur was used as electron donor.

## Introduction

T. denitrificans is found in soil and water habitants. It is not toxic and pathogenic microorganism and it belongs to group of chemolithoautotrophic and facultative anaerobic bacteria. In terms of environmental, T. denitrificans is very important, because this organism can be used in processes of alternative methods of removing nitrates from contaminated water. It can obtain energy by the oxidation of elemental sulfur and certain other sulfur-containing inorganic compounds while electrons are released. Nitrate and nitrite ions are reduced by these electrons and molecules of nitrogen gas are formed under anoxic conditions1-4.

The optimal function of autotrophic denitrification using T. denitrificans is influenced by a number of agents and factors1, 5-7. In this work, we focused on the optimal dosage of phosphorus to the reactor, which is one of the key factors affecting growth of bacteria cells and which does not lead to secondary contamination of treated water.

## Experiments

All experiments were done in batch reactors: ten reactors (with volume 1 l) were prepared for testing the effect of phosphorus at the rate and course of the denitrification processes of bacteria T. denitrificans. All reactors contained crushed sulfur as electron donor (size fraction from 2.5 to 5.0 mm), crushed limestone for pH control (size fraction from 3.0 to 4.0 mm), solution of NaNO3 with initial concentration of 100 mg/l NO3- and suspension of bacterial cells T. denitrificans (optical density of suspension was OD = 0.17).

Table I

Summary of batch experiments and their content

| Label | S, CaCO3 - 1:1 [g] | cNO3- [mg/l] | cP-PO43- [mg/l] |
| --- | --- | --- | --- |
| K1 | 100 | 100 | - |
| K2 | 200 | 100 | - |
| P1 | 100 | 100 | 0.5 |
| P2 | 100 | 100 | 1.0 |
| P3 | 100 | 100 | 1.5 |
| P4 | 100 | 100 | 2.0 |
| P5 | 100 | 100 | 3.0 |
| P6 | 200 | 100 | 0.5 |
| P7 | 200 | 100 | 1.0 |
| P8 | 200 | 100 | 1.5 |

Two reactors was labeled as a control samples; in all other was added phosphorus in the form of NaH2PO4 in various concentration ranges. Summary of batch experiments and their content is shown in Table 1. For growing of Th. denitrificans was used broth S6 – its content is shown in Table II.

The denitrification process in the reactors labeled K1 and P1 – P5 was monitored for 80 days and reactors labeled K2 and P6 – P8 was monitored 40 days. The values of concentrations of nitrate, nitrite and phosphate, pH, temperature, redox potential and dissolved oxygen concentration were measured8-13. During the experiments, it was also verified the presence of bacteria by microscopic techniques.

Table II

Content of S6 broth

| Substance | c [g/l] |
| --- | --- |
| Na2S2O3 | 10.0 |
| KH2PO4 | 11.8 |
| Na2HPO4 | 1.2 |
| MgSO4.7H2O | 0.1 |
| (NH4)2SO4 | 0.1 |
| CaCl2 | 0.003 |
| FeCl3 | 0.02 |
| MnSO4 | 0.02 |

## Result and discussion

The reduction of nitrate and nitrite ions in all reactors (Figure 1 – 4) shows the adaptation phase of bacterial cells to environment (lag phase) in the first 20 days approximately. In these 20 days, there can be seen minimal loss of nitrate ions and formation of a corresponding amount of nitrite ions. There are two explanations for this effect. Firstly, it is because of lag phase, and secondly, in the system there is still remaining portion of oxygen. Oxygen has precedence over nitrate and nitrite ions as a terminal electron acceptor in energy metabolism of T. denitrificans.

Figure 1. Reduction of NO3- in batch experiments K1 and P1 – P5.

Figure 2. Reduction of NO2- in batch experiments K1 and P1 – P5.

Figure 3. Reduction of NO3- in batch experiments K2 and P6 – P8.

After finish of the lag phase, there is a massive reduction of nitrate to nitrite ions. Then the nitrite ions are also degraded. There can be seen the most distinct differences in the rate of reduction of nitrate and nitrite due to the addition of various concentrations of phosphorus – from the tested concentration range from 
0.5 to 3.0 mg/l phosphorus in batch reactors with lower weigh of sulfur and limestone, nitrate reduction occurs most rapidly at concentration range from 1.0 to 2.0 mg/l of added phosphorus.

Figure 4. Reduction of NO2- in batch experiments K2 and P6 – P8.

In the case of reactors labeled K2 and P6-P8 (with higher weigh of sulfur and limestone), there cannot be seen any positive or negative effect of concentrations range of added phosphorus – it is necessary to prolong the experimental period. From the viewpoint of the rate of nitrite reduction, the best initial concentration of added phosphorus was from 1.0 to 1.5 mg/l. At higher or lower addition of phosphorus, there was seen undesirable increased growth of nitrite concentration.

## Conclusions

The experiments have shown the correct dosing of phosphorus during the denitrification using bacteria T. denitrificans for significant speed-up of these processes in batch experiments with elemental sulfur as electron donor. The obtained results from batch experiments with lower weigh of sulfur indicated, that the best initial concentration of added phosphorus was from 1.0 to 1.5 mg/l phosphorus from the viewpoint of the denitrification rate in batch reactors. In the case of batch experiments with higher weigh of sulfur, there cannot be seen any difference - denitrification processes had same rate.

## Acknowledgement

This work was supported by Ministry of Education, Youth and Sports SGSFCHT_2016006 and by Technology Agency of Czech Republic TA04020258.

## References

1.	Oh S.-E., Kim K.-S., Choi H.-C., Cho J., Kim I. S.: Water Sci. Technol. 42 (3-4), 59 (2000)

2. 	Kelly D. P., Wood A. P.: Int J Evol Microbiol 50/2, 547 (2000)

3. 	Beller H. R., Letain T. E., Chakicherla A., Kane S. T., Legler T. C., Coleman M. A.: J Bacteriol 188/19, 7005 (2006)

4. 	Beller H. R., Chain P. S. G., Letain T. E., Chakicherla A., Larimer F. W., Richardson P. M., Coleman M. A., Wood A. P., Kelly D. P.: J Bacteriol 188/4, 1473 (2006)

5. 	Liu L. H., Koenig A.: Proc Biochem 37, 885 (2002)

6. 	Marconi I., Carruci A., Lens P.: Biotech Bioeng 98/3, 551 (2007)

7.	Shao M., Zhang T., Fang H. H.: Appl Microbiol Biotechnol 88/5, 1027(2010)

8.	ČSN ISO 7890-3 (75 7453): Jakost vod. Stanovení dusičnanů s kyselinou sulfosalicylovou. (1994)

9.	ČSN EN 26777 (75 7452). Jakost vod. Stanovení dusitanů. Molekulární absorpční spektrofotometrická metoda (ISO 6777:1984). (1995)

10.	ČSN ISO 10523 (75 7365). Jakost vod – Stanovení pH. (2010)

11.	ČSN ISO 757342 (75 7342). Kvalita vod – Stanovení teploty. (2013)

12.	ČSN ISO 757367 (75 7367). Jakost vod – Stanovení oxidačně redukčního potenciálu (ORP). (2011)

13. ČSN EN ISO 5814 (75 7463). Kvalita vod. Stanovení rozpuštěného kyslíku. Elektrochemická metoda membránovou sondou. (2013)